Robot Autonomy - HW1 - Control

Submission guidelines: Submission of this assignment is in pairs. You must submit a single zip file.

Project setup guide:

1. Install Python

Go to the official Python website: https://www.python.org/downloads/.

Download Python 3.10

During installation, check the box “Add Python to PATH”.

2. Install an Editor

You’ll need a code editor or IDE. Recommended: PyCharm

3. Get the Project Files

Place the following files in a single project folder

4. Install Required Packages

This project depends on numpy and matplotlib. Install them using the terminal command:

pip install numpy matplotlib

5. Run the Simulations

(a) PID Wall Following

python PID_wall_following.py

Q1: PID

You are required to design and tune a PID controller that controls the steering angle of a car to drive it along a wall (keeps 1m from the wall).

- Implement PID class in PID_wall_following.py.

- Set the parameters kp=0.3, kd=0, ki=0. Report the error graph and average error value. Repeat for kp=1.0, kd=0, ki=0. Explain the differences.

- Set the parameters kp=0.3, kd=0, ki=0. Report the error graph and average error value. Repeat for kp=0.3, kd=0.2, ki=0. Explain the differences.

- Now we add constant error to steering execution. Set STEERING_ERROR_CONST = 1.0. and kp=1.5, kd=0.05, ki=0.0. Report the error graph and average error value. Repeat for kp=1.0, kd=0.05, ki=0.5. Explain the differences.

- To account for noise in the real world we assume inaccuracy of localization and steering execution, Set STEERING_ERROR_CONST = 0, STEERING_ERROR_PRECENTAGE = 0.3, and LOCALIZATION_ERROR_METER = 0.5. Report the error graph and average error value. Tune kp, kd, ki as you wish and provide 3 examples. Report the error graph. Suggest a method to tune the parameters kp, kd, ki.

Q2: Pure Pursuit

You are required to implement a pure pursuit controller to track a given path.

- Implement the PurePursuit_controller class in PurePursuit.py. You may change the class and functions as you wish. Do not change other parts of the code.

- Set lookahead distance Lfc=0.3 and report tracking graph and error graph.

- Set lookahead distance Lfc=1.0 and report tracking graph and error graph. Explain the differences with the previous results.

- Suggest a method to tune the lookahead distance parameter, test and explain your method, and provide a tracking and error graph.